Mathematical Description of the Soil Moisture Model

Overview

This document describes the mathematics implemented in the supplied soil moisture model code. The model is a weekly, single-bucket water balance model in which soil water storage is updated from precipitation, canopy interception, infiltration, runoff, evapotranspiration, and drainage. The model also modifies these fluxes using clay content, elevation, biomass, and freeze–thaw status.

The prognostic state variable is soil water storage, denoted by St in mm at weekly time step t. All water fluxes and storages are expressed as equivalent water depths in millimeters over the ground area.

Core state and water balance

Let t=0,1,…,T-1 denote the weekly time index. The model initializes soil moisture as

S0=Sinit

where Sinit is the parameter initial_soil_moisture_mm.

For each week, the soil water balance is updated as

St+1=clipSt+It(soil)-ETa,t-Dt, 0, Smax

where:

- It(soil) is infiltration into the soil,

- ETa,t is actual evapotranspiration,

- Dt is drainage,

- Smax is the effective maximum soil water storage,

- clip restricts storage to the physically allowed interval [0,Smax].

The model therefore behaves as a bucket model with upper and lower bounds on soil water content.

Soil hydraulic properties modified by clay

Clay content modifies four effective soil properties: storage capacity, drainage coefficient, runoff potential, and stress curvature. Let C be clay percentage and Cref the reference clay percentage. Then the clay deviation is

ΔC=C-Cref.

The maximum soil water capacity is

Smax=clipSbase+aawc ΔC, 60, 320

where Sbase is base_soil_water_capacity_mm and aawc is awc_mm_per_clay_pct.

The drainage coefficient is

kdrain=clipkbaseexp⁡(-αdrainΔC), 0.015, 0.25

where kbase is base_drainage_coeff and αdrain is drainage_clay_sensitivity.

The maximum runoff fraction is

frunoff,max=clipfbaseexp⁡(αrunoffΔC), 0.05, 0.90

where fbase is base_runoff_fraction_max and αrunoff is runoff_clay_sensitivity.

The water stress exponent is

p=clip1.4+αstressΔC, 1.0, 2.2

where αstress is stress_clay_sensitivity.

Biomass controls on canopy processes

Biomass Bt controls three canopy-related functions through saturating response curves.

First, the crop or canopy coefficient is

Kc,t=Kc,min+(Kc,max-Kc,min)BtBt+B1/2(Kc)

where kc_min, kc_max, and biomass_half_sat_gC_m2 define the lower bound, upper bound, and half-saturation biomass respectively.

Second, canopy interception capacity is

Imax,t=ImaxBtBt+B1/2(I)

where interception_max_mm is the maximum possible interception and interception_biomass_half_sat_gC_m2 is the half-saturation biomass for interception.

Third, the biomass-dependent uptake factor is

ut=1+(umax-1)BtBt+B1/2(u)

where uptake_max_factor and uptake_biomass_half_sat_gC_m2 control the asymptote and half-saturation point.

These relationships allow more developed vegetation canopies to intercept more rainfall and exert higher evaporative demand.

Potential evapotranspiration

Potential evapotranspiration is estimated from air temperature using a weekly degree-day formulation. For mean weekly temperature Tt, degree-days above a base threshold are

DDt=max(Tt-Tbase,0)×7

where Tbase is temp_base_c.

Base potential evapotranspiration is then

PETtbase=cPET DDt

where cPET is pet_coeff_mm_per_degday.

The model further adjusts PET for elevation using

melev=clip1+αzz-zref1000, 0.75, 1.35

where z is elevation in meters, zref is reference_elevation_m, and αz is elev_pet_sensitivity_per_km.

Thus the elevation-adjusted atmospheric demand is

PETtatm=PETtbase melev.

The canopy-modified potential evapotranspiration is

ETp,t=PETtatmKc,tνt.

Interception, throughfall, runoff, and infiltration

Weekly precipitation Pt is partitioned first by canopy interception:

It=min(Pt,Imax,t)

and throughfall is

Tt=Pt-It.

The soil water store from the previous week is converted to relative saturation

θt=StSmax.

Under unfrozen conditions, the runoff fraction increases with wetness according to

frunoff,t=frunoff,max θtnrunoff

where nrunoff is runoff_exponent.

Runoff and infiltration are then

Rt=frunoff,tTt, It(soil)=Tt-Rt.

This formulation means runoff is small when the soil is dry and increases as the bucket approaches saturation.

Soil water stress and actual evapotranspiration

Before evapotranspiration, available water in the bucket is

Stavail=min(Smax,St+It(soil)).

Water stress is represented as a power-law function of relative storage:

σt=StavailSmaxp

with 0≤σt≤1. Here σt is the quantity stored in the output column water_stress.

Potential evapotranspiration is limited by stress and by actual water availability, giving

ETa,t=minETp,tσt, Stavail.

This ensures evapotranspiration cannot remove more water than is available in storage.

Drainage and field capacity

The model represents field capacity as a constant fraction of the total storage capacity:

SFC=fFCSmax

where fFC is field_capacity_fraction.

After evapotranspiration, intermediate storage is

StpostET=Stavail-ETa,t.

If this exceeds field capacity, gravitational drainage occurs:

Dt=kdrainStpostET-SFC,StpostET>SFC0,StpostET≤SFC

so drainage only affects the water held above the field-capacity threshold.

Freeze–thaw representation

The model includes a simple temperature-triggered freeze–thaw routine. Consecutive weeks with mean temperature below 0 °C are counted. When the number of consecutive freezing weeks exceeds freeze_trigger_weeks, the soil is flagged as frozen. The soil remains frozen until weekly mean temperature rises above thaw_trigger_temp_c.

When the soil is frozen, several hydrological processes are modified:

- drainage is reduced by multiplying the drainage coefficient by frozen_drainage_multiplier,

- only a fraction frozen_infiltration_fraction of throughfall infiltrates and the remainder becomes runoff,

- potential evapotranspiration is reduced by frozen_pet_multiplier,

- biomass-enhanced uptake is damped by frozen_uptake_multiplier.

Thus frozen conditions reduce infiltration, limit drainage and evapotranspiration, and generally increase runoff.

Parameter table

| Parameter name | Description | Units |
| --- | --- | --- |
| initial_soil_moisture_mm | Initial soil moisture storage at the start of the simulation | mm |
| base_soil_water_capacity_mm | Base soil water holding capacity before clay adjustment | mm |
| base_drainage_coeff | Base drainage coefficient controlling drainage above field capacity | fraction |
| base_runoff_fraction_max | Maximum runoff fraction under very wet conditions before clay adjustment | fraction |
| runoff_exponent | Exponent controlling how strongly runoff increases with relative saturation | dimensionless |
| field_capacity_fraction | Field capacity expressed as a fraction of maximum soil water capacity | fraction |
| reference_clay_pct | Reference clay content used as the baseline for soil adjustments | % |
| awc_mm_per_clay_pct | Change in water capacity per percentage point of clay | mm per % clay |
| drainage_clay_sensitivity | Sensitivity of the drainage coefficient to clay content | 1 per % clay |
| runoff_clay_sensitivity | Sensitivity of maximum runoff fraction to clay content | 1 per % clay |
| stress_clay_sensitivity | Sensitivity of the soil water stress exponent to clay content | 1 per % clay |
| temp_base_c | Base temperature threshold used in the PET calculation | °C |
| pet_coeff_mm_per_degday | PET coefficient converting degree-days to weekly PET | mm per °C day |
| reference_elevation_m | Reference elevation for PET adjustment | m |
| elev_pet_sensitivity_per_km | Sensitivity of PET to elevation difference | per km |
| kc_min | Minimum crop or canopy coefficient at very low biomass | dimensionless |
| kc_max | Maximum crop or canopy coefficient at high biomass | dimensionless |
| biomass_half_sat_gC_m2 | Half-saturation biomass for the crop coefficient response | gC m^-2 |
| interception_max_mm | Maximum canopy interception capacity | mm |
| interception_biomass_half_sat_gC_m2 | Half-saturation biomass for interception capacity | gC m^-2 |
| uptake_max_factor | Maximum biomass-driven uptake multiplier | dimensionless |
| uptake_biomass_half_sat_gC_m2 | Half-saturation biomass for the uptake multiplier | gC m^-2 |
| freeze_trigger_weeks | Number of consecutive weeks below 0 °C needed to trigger frozen soil status | weeks |
| thaw_trigger_temp_c | Temperature threshold for thawing frozen soil | °C |
| frozen_infiltration_fraction | Fraction of throughfall allowed to infiltrate when the soil is frozen | fraction |
| frozen_drainage_multiplier | Multiplier applied to drainage coefficient under frozen conditions | fraction |
| frozen_uptake_multiplier | Multiplier applied to biomass-enhanced uptake under frozen conditions | fraction |
| frozen_pet_multiplier | Multiplier applied to PET under frozen conditions | fraction |

Output variables

The model returns a DataFrame containing both the forcing data and diagnostic outputs. Key outputs are final weekly soil moisture (soil_moisture_mm), relative saturation (relative_soil_moisture), potential and actual evapotranspiration (et_potential_mm, et_actual_mm), runoff (runoff_mm), drainage (drainage_mm), interception (intercepted_mm), throughfall (throughfall_mm), water stress (water_stress), and the frozen-soil indicator (frozen_soil).

Overall, the code represents a compact ecohydrological water balance model in which precipitation is partitioned by canopy interception and runoff, evapotranspiration is driven by temperature and biomass, soil storage is controlled by clay-modified capacity, and cold-period effects are represented by a simple freeze–thaw switch.

| Output column | Mathematical quantity |
| --- | --- |
| soil_moisture_mm | St+1 |
| relative_soil_moisture | θt+1=St+1/Smax |
| et_actual_mm | ETa,t |
| et_potential_mm | ETp,t* |
| runoff_mm | Rt |
| drainage_mm | Dt |
| intercepted_mm | It |
| throughfall_mm | Tt |
| water_stress | σt |
| frozen_soil | Ft |